1. Implement Edge Detection Using Sobel Templates

- Sobel Vertical

- Description:

Vertical template:

By comparing a 3x3 pixel area in the image with each template, the system assesses the similarity of this region to the provided template. In the context of vertical edge detection, a high value indicates that the template has identified a vertical edge in the image.

Nedderman Hall

Chess board

- Sobel Horizontal

- Description:

Horizontal template:

By comparing a 3x3 pixel area in the image with each template, the system assesses the similarity of this region to the provided template. In the context of horizontal edge detection, a high value indicates that the template has identified a horizontal edge in the image.

Nedderman Hall

Chess board

- Sobel Major Diagonal

- Description:

Major Diagonal template:

By comparing a 3x3 pixel area in the image with each template, the system assesses the similarity of this region to the provided template. In the context of major diagonal edge detection, a high value indicates that the template has identified a major diagonal edge in the image.

Nedderman Hall

Chess board

- Sobel Minor Diagonal

- Description:

Minor Diagonal template:

By comparing a 3x3 pixel area in the image with each template, the system assesses the similarity of this region to the provided template. In the context of minor diagonal edge detection, a high value indicates that the template has identified a minor diagonal edge in the image.

Nedderman Hall

Chess board

Code:
#include <stdio.h>

#include <math.h>

#include <X11/Xlib.h>

#define DIM 512

/******************************************************************/

/* This structure contains the coordinates of a box drawn with    */

/* the left mouse button on the image window.                     */

/* roi.x , roi.y  - left upper corner's coordinates               */

/* roi.width , roi.height - width and height of the box           */

/******************************************************************/

extern XRectangle roi;

unsigned char convolution(unsigned char image[DIM][DIM],int template[3][3],int row,int col);

/******************************************************************/

/* Main processing routine. This is called upon pressing the      */

/* Process button of the interface.                               */

/* image  - the original greyscale image                          */

/* size   - the actual size of the image                          */

/* proc_image - the image representation resulting from the       */

/*              processing. This will be displayed upon return    */

/*              from this function.                               */

/******************************************************************/

void process_image(image, size, proc_img)

unsigned char image[DIM][DIM];

int size[2];

unsigned char proc_img[DIM][DIM];

{

// Sobel templates

// Please uncomment the desired template as a parameter in the convolution function

// int sobelVertical[3][3] = {{-1, 0, 1}, {-2, 0, 2}, {-1, 0, 1}};

// int sobelHorizontal[3][3] = {{-1, -2, -1}, {0, 0, 0}, {1, 2, 1}};

// int sobelMajDiag[3][3] = {{0, -1, -2}, {1, 0, -1}, {2, 1, 0}};

int sobelMinDiag[3][3] = {{-2, -1, 0}, {-1, 0, 1}, {0, 1, 2}};

for (int x = 0; x < size[0] - 2; x++)

{

for (int y = 0; y < size[1] - 2; y++)

{

proc_img[x][y] = convolution(image,sobelMinDiag,x,y); // switch the second parameter, if needed.

}

}

}

unsigned char convolution(unsigned char image[DIM][DIM],int template[3][3],int row,int col)

{

int sum = 0;

for (int i = 0; i < 3; i++)

{

for (int j = 0; j < 3; j++)

{

sum += (int)image[row + i][col + j] * template[i][j];

}

}

// check if sum is out of range [0:255]

if (sum < 0)

{

sum = 0;

}

else if (sum > 255)

{

sum = 255;

}

return (unsigned char)sum;

}

2. Implement Template Matching Using Normalized Convolution

- Description:

Formula used:

Normalized Convolution calculates the cross-correlation between a portion of an image and a selected template, providing a measure of similarity. Both the image and the template need to undergo normalization before the cross-correlation process. The brighter the area, the more similar it is to the template.

- Nedderman Hall

Template:

Result:

- Chess board

Template:

Result:

Code:

#include <stdio.h>

#include <math.h>

#include <X11/Xlib.h>

#define DIM 512

extern XRectangle roi;

float convolution(unsigned char image[DIM][DIM], float template[roi.width][roi.height], int row, int col, float template_mean, float template_stddev);

void normalize(float data[roi.width][roi.height], float mean, float stddev);

void get_mean_stddev(float data[roi.width][roi.height], float *mean, float *stddev);

void process_image(unsigned char image[DIM][DIM], int size[2], unsigned char proc_img[DIM][DIM])

{

// selected templates

float template[roi.width][roi.height];

float template_mean, template_stddev;

// extracting template from image

for (int x = 0; x < roi.width; x++)

{

for (int y = 0; y < roi.height; y++)

{

template[x][y] = (float)image[roi.x + x][roi.y + y];

}

}

get_mean_stddev(template, &template_mean, &template_stddev);

normalize(template, template_mean, template_stddev);

for (int x = 0; x < size[0]-roi.width; x++)

{

for (int y = 0; y < size[1]-roi.height; y++)

{

float conv_result = convolution(image, template, x, y, template_mean, template_stddev);

// Normalize the result of the convolution to be between 0 and 255

proc_img[x + roi.width / 2][y + roi.height / 2] = (unsigned char)(conv_result * 255.0f);

}

}

}

float convolution(unsigned char image[DIM][DIM], float template[roi.width][roi.height], int row, int col, float template_mean, float template_stddev)

{

float sum = 0.0;

float n = roi.height * roi.width;

float subimage_mean, subimage_stddev;

float subimage[roi.width][roi.height];

// Extract subimage and calculate its mean and standard deviation

for (int x = 0; x < roi.width-1; x++)

{

for (int y = 0; y < roi.height-1; y++)

{

subimage[x][y] = (float)image[row+x][col+y];

}

}

get_mean_stddev(subimage, &subimage_mean, &subimage_stddev);

normalize(subimage, subimage_mean, subimage_stddev);

// Perform normalized cross-correlation

for (int i = 0; i < roi.width; i++)

{

for (int j = 0; j < roi.height; j++)

{

sum += subimage[i][j] * (template[i][j] - template_mean) / template_stddev;

}

}

sum = fmax(0.0, fmin(sum / n, 1.0)) * 255.0;

return sum;

}

void normalize(float data[roi.width][roi.height], float mean, float stddev)

{

// Normalize each pixel value using the mean and standard deviation

for (int i = 0; i < roi.width; i++)

{

for (int j = 0; j < roi.height; j++)

{

data[i][j] = (data[i][j] - mean) / stddev;

}

}

}

void get_mean_stddev(float data[roi.width][roi.height], float *mean, float *stddev)

{

*mean = 0.0;

*stddev = 0.0;

// Compute mean

for (int i = 0; i < roi.width; i++)

{

for (int j = 0; j < roi.height; j++)

{

*mean += data[i][j];

}

}

*mean /= (roi.width * roi.height);

// Compute standard deviation

for (int i = 0; i < roi.width; i++)

{

for (int j = 0; j < roi.height; j++)

{

*stddev += pow(data[i][j] - *mean, 2);

}

}

*stddev /= (roi.width * roi.height);

*stddev = sqrt(*stddev);

}